Q1:

By plotting the Euclidean error between the ground truth angles, and the KNN-predicted joint angles at varying K from 1-20, we can observe the trend as we increase K. At K of 1, we have the sharpest difference between ground truth angle and predicted angle at above 2. The moment we changed K to 2, the difference sharply decreased to above 1, and was at the lowest point for K = 3 under 1. At K = 4, it’s roughly around the same bottom dip above 1, but then as K increases slowly increases and K seems to stabilise around 1.3. This points to K = 3 being the optimal value and the best trade-off for this query.

Very small K values in this case are highly sensitive local samples, making it lack robustness, and can potentially be a noisy estimate a prediction, which explains the large error. Very large K, give another effect where the larger the K, the more likely the prediction is smothered by the less relevant samples, resulting in the gradual increase in error we observe as K increases above 4. So, a K that’s too small leads to high variance where the estimator is sensitive to small data, but a K that’s too large ends up potentially underfitting or having a high bias.

Ultimately, prediction accuracy is best at an intermediate K, where the model balances local fidelity and robustness, which is observed at the minimum point of the K-error curve. In this case, the point is K=3.

Q3:

k_att controls how strongly the robot is pulled toward the goal. When k_att is too small, the attractive pull is weak, resulting in slow, inefficient paths that may not converge (seen at k_att = 0.1). When k_att is too large, the pull becomes aggressive, causing overshooting and oscillation around the goal while even cutting through obstacles (k_att = 2.0+).

k_rep controls how strongly the robot is pushed away from obstacles. When k_rep is too small, repulsion is insufficient, allowing the robot to pass dangerously close to obstacles for a more direct but unsafe path (k_rep = 0.001). When k_rep is too large, the repulsive force becomes overwhelming, causing the robot to get trapped in local minima as it oscillates between obstacles with no viable escape route (k_rep = 0.05+).

The choice of k_att and k_rep depends on the environment structure. In tight spaces with narrow passages, aggressive tuning fails—high k_rep causes oscillation-induced local minima, while low k_att fails to provide sufficient pull through constraints. Conversely, in wide, open environments, both coefficients can be increased to achieve faster convergence and safer paths with wider safety margins.